SIMATS ENGINEERING

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | KAVALI BALAJI | Reg.No.: | 192525402 |
| Department: | AI&ml | Date: | 29-08-2026 |

ANNEXURE A
COMPARATIVE ANALYSIS

1. Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

2. Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

3. Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

4. Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

5. Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I KAVALI BALAJI (192525402)_________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

Q1. Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

1. Overview / Scope
Modern processors use a multi-level cache hierarchy (L1, L2, L3) between the CPU core and main memory (DRAM) to reduce the effect of the speed difference between the processor and main memory. The three cache levels differ in proximity, capacity, speed, and sharing. This comparison considers latency, bandwidth, throughput, and their effect on processor performance.

| Parameter | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Inside each core (private) | Per-core or per-cluster (private) | Shared across all cores (on-die) |
| Typical Size | 32–64 KB (per core, split I/D) | 256 KB – 1 MB (per core) | 8 MB – 64 MB (shared) |
| Access Latency | ~1–4 CPU cycles (~1 ns) | ~10–12 cycles (~3–5 ns) | ~30–40 cycles (~10–20 ns) |
| Bandwidth | Highest (several hundred GB/s per core) | High, lower than L1 (100–200 GB/s) | Moderate, shared among cores (50–100 GB/s aggregate) |
| Throughput | Very high per-core instruction/data supply | High, slightly reduced because of the larger array | Lower per access, but large capacity supports bulk transfers |
| Technology | SRAM, fastest and most expensive per bit | SRAM, slightly denser/slower than L1 | SRAM (sometimes eDRAM), largest and slowest of the three |
| Hit Rate Contribution | Filters most repeated/loop accesses | Captures medium working-set data missed by L1 | Captures large working sets shared across cores and reduces DRAM traffic |

3. Analytical Discussion
L1 cache is divided into instruction and data caches and is located closest to the execution units. It is designed for the minimum possible latency, so its small size allows access within only a few cycles. An L1 miss can stall the processor pipeline. L2 cache is larger and normally unified, so it stores a bigger working set while accepting greater latency. This reduces the number of accesses that need to go to L3 or DRAM. L3 cache is shared by the processor cores, has the largest capacity, and works as the last cache level before main memory. Its higher latency is acceptable because it is accessed only after L1 and L2 misses. Its shared structure also supports inter-core data sharing and cache coherence.

Bandwidth generally decreases as the cache level moves farther from the processor because lower levels have greater distance and more competing consumers. Throughput follows a similar pattern, but hit rate is also important. A properly sized L2 or L3 can maintain good effective throughput because it avoids expensive accesses to DRAM.

4. Impact on Processor Performance
● Average Memory Access Time (AMAT) is strongly influenced by the L1 hit rate. Even a small improvement in L1 hit rate can improve instructions-per-cycle (IPC), since L1 misses occur on the critical path of instruction execution.

● L2 works as a latency and bandwidth buffer. It handles many L1 misses, allowing only a smaller fraction of requests to reach L3 or DRAM. This helps maintain pipeline throughput in out-of-order processors.

● L3 mainly influences multi-core scalability and worst-case memory latency. A larger and effectively managed L3 reduces DRAM bandwidth pressure and inter-core coherence traffic, which is important for multi-threaded and server workloads.

● Overall performance can be expressed using the AMAT relationship:
AMAT = HitTime(L1) + MissRate(L1) × [HitTime(L2) + MissRate(L2) × (HitTime(L3) + MissRate(L3) × MemoryAccessTime)].
This shows how latency and hit rate at each cache level combine through the hierarchy.

5. Conclusion
L1, L2, and L3 provide a planned balance between speed, capacity, and cost. L1 is optimized for minimum latency, L2 provides a middle ground between speed and capacity, and L3 emphasizes larger capacity and sharing. Together, these levels allow modern processors to obtain the performance benefits of a large fast memory system while using only a limited amount of expensive high-speed memory.

Q2. Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

1. Overview / Scope
SRAM (Static RAM) and DRAM (Dynamic RAM) are the two major volatile memory technologies in a computer memory hierarchy. SRAM is mainly used for on-chip cache levels such as L1, L2, and L3, while DRAM is mainly used for main memory. Their different cell structures create differences in latency, bandwidth, throughput, density, power, and cost.

| Parameter | SRAM | DRAM |
| --- | --- | --- |
| Cell Structure | 6-transistor (6T) flip-flop, no capacitor | 1 transistor + 1 capacitor (1T-1C) |
| Refresh Requirement | Not required while power is supplied | Periodic refresh required (~64 ms cycle) because of capacitor leakage |
| Latency | Very low (~1–2 ns, single-digit cycles) | Higher (~50–100 ns, tens of cycles) due to row activation/precharge |
| Bandwidth | Very high, mainly limited by on-chip wiring | High aggregate bandwidth (e.g., DDR5 ~40–50 GB/s per channel), but lower per access than SRAM |
| Throughput | High sustained throughput for small, frequent accesses | High burst throughput for large sequential transfers; lower for random small accesses |
| Density / Cost per bit | Low density, high cost per bit | High density, low cost per bit |
| Power Consumption | Higher static leakage power; no refresh power | Lower per-cell static power, but refresh adds dynamic power |
| Typical Use | L1/L2/L3 caches, register files | Main memory (DIMMs), frame buffers |

3. Analytical Discussion
SRAM uses a six-transistor cell that retains its state while power is available, so it does not require the refresh operations needed by DRAM. This gives SRAM very fast and predictable access, making it suitable for caches where low latency directly supports instruction throughput. DRAM uses a one-transistor/one-capacitor cell, which is denser and cheaper per bit. However, capacitor charge leaks over time, so refresh is required. Row activation and column access operations also contribute to its greater latency.

For bandwidth, SRAM can provide very fast random access and its performance is largely constrained by the on-chip interconnect. DRAM can provide high peak bandwidth through burst transfers, wide interfaces, and technologies such as DDR, GDDR, and HBM. However, its bandwidth can decrease with random, small, or bank-conflicting requests.

4. Role in Cache vs. Main Memory Design
● Cache design uses SRAM because its low and predictable latency helps keep the CPU pipeline supplied with data. Its high cost and lower density are acceptable because cache capacity is intentionally limited.

● Main memory uses DRAM because it provides high density and lower cost per bit, making it practical to provide gigabytes of storage. The higher latency is acceptable because effective cache operation reduces how often the CPU accesses DRAM.

● Overall throughput depends on the combination of both technologies. SRAM caches handle most frequent accesses, while DRAM provides the large capacity required for the complete working set and supplies data after cache misses.

5. Conclusion
SRAM and DRAM are selected for different purposes. SRAM provides very high speed at relatively small capacity and is therefore suitable for processor caches. DRAM provides greater capacity at a lower cost per bit and is therefore suitable for main memory. Their latency and bandwidth differences arise from their cell structures, and an efficient memory hierarchy uses both technologies together.

Q3. Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

1. Overview / Scope
Cache memory and virtual memory with paging are both hierarchical techniques, but they operate at different levels and address different requirements. Cache memory reduces the latency gap between the CPU and DRAM, while virtual memory maps a process's logical address space to physical memory and can use disk or SSD storage as backing space when necessary.

| Parameter | Cache Memory Mechanism | Virtual Memory (Paging) Mechanism |
| --- | --- | --- |
| Purpose | Reduces the CPU–main-memory latency gap | Extends usable address space beyond physical RAM and provides isolation/protection |
| Managed By | Hardware (cache controller) | Hardware (MMU/TLB) + Operating System |
| Unit of Transfer | Cache line (typically 64 bytes) | Page (typically 4 KB, or larger with huge pages) |
| Hit Latency | Nanoseconds (L1: ~1 ns, L3: ~10–20 ns) | TLB hit: ~1 cycle; TLB miss page-table walk: tens of cycles |
| Miss Penalty | Access to the next cache level or DRAM (tens to ~100 ns) | Page fault requiring disk/SSD: milliseconds (SSD) or ~10 ms (HDD) |
| Throughput Impact | High; misses are relatively inexpensive and can occur frequently | Low miss tolerance; frequent page faults can greatly reduce throughput |
| Replacement Policy | LRU/Pseudo-LRU implemented in hardware | LRU approximation such as clock algorithm implemented by the OS |

3. Analytical Discussion
Both mechanisms use locality, hit/miss behavior, and replacement policies, but their miss penalties are very different. A cache miss normally costs tens to around one hundred nanoseconds because the request moves to another cache level or DRAM. A virtual-memory page fault can take milliseconds because secondary-storage I/O is required. Therefore, the difference in latency can be several orders of magnitude. Cache systems can tolerate relatively frequent misses, whereas virtual-memory systems must keep page-fault rates very low through adequate physical memory, working-set management, and suitable prefetching.

The Translation Lookaside Buffer (TLB) is a small, fast cache for page-table entries. A TLB hit makes virtual-to-physical address translation very fast. A TLB miss requires a page-table walk and, in the worst case, may be followed by a page fault, showing how virtual-memory performance is connected to cache-like structures.

4. Impact on System Throughput
● Cache throughput depends strongly on hit rates at each cache level. The L1→L2→L3→DRAM hierarchy absorbs many misses without immediately causing very large delays.

● Virtual-memory throughput is highly dependent on the page-fault rate. Excessive page faults, or thrashing, can severely reduce system throughput because the CPU waits for slow secondary-storage operations.

● Both mechanisms exploit temporal and spatial locality. Cache memory works transparently at nanosecond timescales, whereas virtual memory operates with larger pages and operating-system involvement, providing flexibility, protection, and support for programs whose logical address spaces exceed physical RAM.

5. Conclusion
Cache memory and virtual-memory paging are complementary mechanisms. Cache memory focuses on reducing CPU access latency across the CPU–DRAM gap, while virtual memory manages capacity and protection across the DRAM–storage gap. High cache hit rates and low page-fault rates are both important for maintaining high overall system throughput.

Q4. Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

1. Overview / Scope
NAND Flash and DRAM occupy different positions in a memory and storage hierarchy. DRAM is normally used as volatile main memory, while NAND Flash is commonly used for non-volatile secondary storage such as SSDs and can also serve in lower storage-class tiers. Their comparison shows why they are suited to different roles.

| Parameter | NAND Flash | DRAM |
| --- | --- | --- |
| Volatility | Non-volatile; retains data without power | Volatile; loses data without power |
| Cell Structure | Floating-gate/charge-trap transistor storing charge as bits | 1T-1C capacitor-based cell |
| Read Latency | ~25–100 µs (microseconds) | ~50–100 ns (nanoseconds), roughly 1000× faster than NAND |
| Write/Program Latency | ~200–900 µs, plus block erase-before-write overhead | ~50 ns, with no erase requirement |
| Bandwidth | Sequential: ~500 MB/s–7 GB/s (NVMe SSD); random access much lower | Tens of GB/s per channel (DDR5 ~40–50 GB/s); high for sequential and random access |
| Endurance | Limited program/erase cycles (~1,000–100,000 depending on type) | Effectively unlimited write endurance |
| Granularity | Read/write in pages (~4–16 KB); erase in blocks (~MB) | Read/write at byte/word granularity |
| Cost per Bit | Low | Higher than NAND Flash |
| Role in Hierarchy | Secondary storage / storage-class memory tier | Primary (main) memory |

3. Analytical Discussion
The main reason for the performance difference is the physical storage mechanism. DRAM cells are accessed electrically at nanosecond speeds and do not have a wear mechanism. NAND Flash stores charge in a floating gate or charge-trap structure and requires sensing and programming operations, which take longer and introduce wear. NAND also requires block erase before rewriting pages, whereas DRAM can be overwritten directly.

Modern NVMe SSDs can deliver high sequential bandwidth by using many channels and flash dies in parallel. However, DRAM remains much better for random-access performance and predictable latency. NAND controllers can also introduce additional latency variation through the flash translation layer and garbage collection.

4. Suitability for Hierarchical Memory Systems
● DRAM is suitable as main memory because its nanosecond latency, fine-grained addressing, and high endurance match the CPU's need for rapid access to active data.

● NAND Flash is suitable for persistent secondary storage because it is non-volatile, has a lower cost per bit than DRAM, and provides large capacity for data that is less latency-sensitive.

● In newer hierarchical designs, NAND Flash or NVMe-based storage tiers can support caching or swap functions below DRAM. This can increase effective capacity while still being much faster than traditional hard-disk storage, although it remains considerably slower than DRAM.

5. Conclusion
NAND Flash and DRAM are complementary rather than interchangeable. DRAM provides the fast, byte-level working memory required close to the processor, while NAND Flash provides persistent, dense, lower-cost storage farther down the hierarchy. Their combination supports virtual-memory paging and modern tiered-storage systems.

Q5. Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

1. Overview / Scope
MRAM (Magnetoresistive RAM) and RRAM (Resistive RAM or ReRAM) are emerging non-volatile memory technologies intended to reduce the gap between fast but volatile SRAM/DRAM and dense but slower NAND Flash. Both provide non-volatility and relatively fast operation, but their physical storage mechanisms lead to different performance characteristics.

| Parameter | MRAM (Magnetoresistive RAM) | RRAM (Resistive RAM) |
| --- | --- | --- |
| Storage Mechanism | Magnetic tunnel junction (MTJ); data stored using magnetic/electron-spin orientation | Resistance switching in a metal-oxide layer through formation and rupture of conductive filaments |
| Access Time | Very fast, ~2–20 ns (STT-MRAM), close to DRAM/SRAM | Moderate-fast, ~10–50 ns, generally slower than MRAM |
| Write Endurance | Very high (~10^12–10^15 cycles), near-DRAM class | Moderate (~10^6–10^12 cycles), lower than MRAM |
| Throughput | High; suitable for cache/DRAM-replacement roles | Good, but somewhat lower sustained throughput because of slower switching/settling |
| Energy Efficiency | Low read energy; moderate write energy due to current-driven switching | Very low read/write energy per bit; variability can add error-correction overhead |
| Non-volatility | Yes; retains data without power | Yes; retains data without power |
| Scalability/Density | Moderate; MTJ stack limits extreme scaling | High; simple two-terminal structure supports scaling and 3D crossbar stacking |
| Maturity/Use Case | Used in embedded NVM and some cache/last-level applications | Promising for storage-class memory and neuromorphic/in-memory computing |

3. Analytical Discussion
MRAM stores information through the relative magnetic orientation of layers in a magnetic tunnel junction. In STT-MRAM, changing this magnetic state allows fast switching and high endurance because the memory state is changed without the same physical wear mechanism found in resistive filament switching. RRAM instead changes resistance by forming and rupturing a conductive filament in a metal-oxide material. This makes RRAM more variable and generally gives it lower endurance, although its simple two-terminal structure supports high density and 3D crossbar integration.

Both technologies can benefit from very low standby power because they are non-volatile. RRAM generally offers very low energy per bit for switching and is attractive for dense, low-power applications. MRAM provides a strong balance of speed, reliability, and endurance, making it more suitable for frequently written, performance-sensitive roles.

4. Comparative Performance Summary
● Access time: MRAM is generally faster and more consistent, with latency closer to DRAM, while RRAM is also fast but may show greater cycle-to-cycle variation.

● Throughput: MRAM is better suited to high-frequency cache-like or DRAM-adjacent applications. RRAM is more suitable for high-density storage-class applications where density is more important than maximum speed.

● Energy efficiency: RRAM generally provides lower per-bit read/write energy, while MRAM offers fast and durable operation with moderate energy use and no refresh requirement.

● Endurance and reliability: MRAM generally has higher endurance and is better for frequently changing data. RRAM's density advantage makes it attractive for larger-capacity applications with less frequent rewriting.

5. Conclusion
MRAM and RRAM are promising non-volatile technologies that can reduce the trade-off between persistence and performance in hierarchical memory systems. MRAM emphasizes fast access and high endurance, making it suitable for roles near the DRAM/cache region. RRAM emphasizes density and energy efficiency, making it attractive for storage-class memory and other high-density applications.